Part a: The ripples are satisfied, as evident from the plots. The output voltage is higher than 24V, which is caused by the turns ratio calculation in the first part of the homework.

Part b: Transformer leakage inductance causes voltage spikes, ringing, increased switching losses, EMI issues, and consequently, deviation from ideal analytical results. Therefore, leakage inductance must be minimized through careful transformer design and managed using snubber or clamp circuits to ensure reliable and efficient converter operation.

Part c: Choose:

MOSFET: Nexperia GANE3R9-150QBA (150V)

Diode : Diodes Inc. SBR20U50SLP-13 (50V, 20A)

Primary side Voltage: 100V
Primary side Current: 3.07A
Secondary side Voltage: 26.7V
Secondary side Current: 11.5A

For the MOSFET:

Switching Loss (Coss = 0.9nF): ½*Coss*Vp^2*fs = 0.9 W

Conduction Loss (RDSon = 3.9mΩ): Iprms^2*R = 0.0165 W (Iprms = Ip*D^0.5)

Temperature (Coefficient: 57.56 °C/W): 40 + 57.56*0.90165 = 93.9°C – No heatsink required!

For the Diode:

Switching Loss (trr = 57ns): Is/2*trr*1/2*Vs*fs = 0.875W

Conduction Loss (Vf = 0.46 V): Vf*Is/2 (Iavg) = 2.645 W

Temperature (Coefficient: 30 °C/W): 40 + 30*3.52 = 145.6°C – Exceeds 125°C – Use Heatsink

If we use heatsink, then the coefficient is 5 °C/W. New Temperature: 40 + 5*3.52 = 57.6 °C – The junction temperature is not exceeded.

Part d: The simplest choice for bidirectional isolated DC-DC would be the Dual-Active-Bridge (DAB). A full H-bridge on the primary, a full H-bridge on the secondary, and a transformer between them. We can control the power flow by changing the phase shift between the two bridges.